SC-VOX

Star Citizen Voice Operations eXchange

Project Specification — Version 1.1

REVISION SUMMARY v1 → v1.1

| Change | Detail |
| --- | --- |
| LED table — added missing entry | GREEN (mic) = Mic Active / Live added to LED color scheme |
| Activity flash LED removed | Replaced by dedicated TX Lamp — brighter sustained indicator, no flash needed |
| TX Lamp added per channel | Larger illuminated lamp, stays lit while channel transmission is active |
| Dedicated power section added | Section 6.3 — standalone 5V supply required, USB insufficient |
| SRS licensing clarified | Section 9.4 — MIT open source, fork permitted, SC-specific fork viable |

# 1. System Overview

SC-VOX is a dedicated hardware voice communications panel designed for Star Citizen multi-crew and multi-org operations. It provides physical, tactile control over up to 6 simultaneous communication channels via Simple Radio Standalone (SRS), in-game VOIP, and Discord, with real-time LED status readout and per-channel audio management.

The panel is designed to complement the SC-UCC (Star Citizen Ultra-Cockpit Controller) button box, sharing the same distributed processing architecture, UART communication protocol, and remote firmware update system.

NOTE: SC-VOX is inspired by real aircraft Audio Control Panels (ACP) and military radio stack designs. Guard channel behavior follows real aviation doctrine — fixed frequency, always monitoring, cannot be disabled.

# 2. Design Philosophy

## 2.1 Real Aviation Doctrine

SC-VOX borrows directly from real aircraft comms panel design:

- Guard channel is fixed, always-on, and non-configurable — exactly as in real aviation

- Per-channel OFF / MON / ACTIVE positional switches mirror real ACP design

- Separate mic and speaker mute controls per channel

- Standby frequency pre-tuning with flip-flop swap

- TX Lamp illuminates and stays lit while a channel is actively transmitting

- PTT/VOX toggle per channel for flexibility between formal and casual nets

## 2.2 Immersion First

Every design decision prioritizes immersion and realistic operational feel:

- Guard channel PTT lives on a dedicated joystick button — never on the panel

- Local/proximity channel uses in-game VOIP bindings — not SRS

- No recording controls on hardware — OBS handles all recording and mixing

- Per-channel OBS audio track integration via OBS Websocket

## 2.3 OBS Integration

SC-VOX integrates with OBS Studio for professional multi-track recording:

| OBS Track | Source | Purpose |
| --- | --- | --- |
| Track 1 | My Mic | Always recorded — pilot voice |
| Track 2 | CH 1 Wing | Wing / flight comms |
| Track 3 | CH 2 Squad | Squad comms |
| Track 4 | CH 3 Org | Org command net |
| Track 5 | CH 4 Multi-Org | Cross-org command |
| Track 6 | CH 5 Guard | Guard channel (always on) |
| Track 7 | CH 6 Local | In-game proximity VOIP |
| Track 8 | Game Audio | SC game audio only |

Post-production editors can select which comms tracks appear in the final video, mute irrelevant chatter, and feature specific channel moments independently.

# 3. Channel Definitions

| CH | Net Name | System | Type | Notes |
| --- | --- | --- | --- | --- |
| 1 | Wing | SRS | Full controls | Immediate crew — 2-4 pilots |
| 2 | Squad | SRS | Full controls | Squad tactical net |
| 3 | Org | SRS | Full controls | Organization command net |
| 4 | Multi-Org | SRS | Full controls | Cross-org coordination |
| 5 | GUARD | SRS | Fixed / passive | Always on — cannot be disabled or tuned |
| 6 | Local | In-game VOIP | Mute controls only | Proximity based — no tuning |

# 4. Per-Channel Feature Matrix

CHANGE v1→v2: Activity flash LED removed. Replaced by TX Lamp — a larger, brighter, sustained indicator that stays lit while transmission is active on that channel.

| Feature | CH 1-4 (Full) | CH 5 Guard | CH 6 Local |
| --- | --- | --- | --- |
| OLED Display | ✅ Active/Standby freq | ✅ GUARD (static) | ✅ LOCAL (static) |
| 3-Position Switch (OFF/MON/TX) | ✅ | ❌ | ❌ |
| PTT / VOX Toggle | ✅ | ❌ PTT only | ❌ PTT only |
| Flip-Flop Button (Active ↔ Standby) | ✅ | ❌ | ❌ |
| Volume Encoder + Push | ✅ | ❌ | ❌ |
| Mic Mute Button | ✅ | ❌ | ✅ |
| Speaker Mute Button | ✅ | ❌ | ✅ |
| MON LED (Yellow) | ✅ | ✅ Always lit | ✅ Always lit |
| ACTIVE LED (Green) | ✅ | ❌ | ❌ |
| Mic Live LED (Green) | ✅ | ❌ | ✅ |
| Mic Muted LED (Red) | ✅ | ❌ | ✅ |
| Speaker Active LED (Blue) | ✅ | ❌ | ✅ |
| Speaker Muted LED (Red) | ✅ | ❌ | ✅ |
| TX Lamp (sustained, brighter) | ✅ | ✅ | ✅ |
| PTT Location | Left D-Pad 2 | Dedicated stick button | Left Pinky button |

# 5. LED Color Scheme

CHANGE v1→v2: GREEN (mic) = Mic Active/Live added. TX Lamp replaces Activity Flash LED.

All LEDs follow a consistent color language for instant status readout:

| LED Color | Meaning | Location |
| --- | --- | --- |
| GREEN (mic) | Mic live / active — they can hear you | Mic section |
| RED (mic) | Mic muted — cannot transmit | Mic section |
| BLUE | Speaker active — you can hear them | Speaker section |
| RED (speaker) | Speaker muted — deaf to channel | Speaker section |
| YELLOW | MON mode — listening only, TX locked | Switch section |
| GREEN (switch) | ACTIVE mode — full participation | Switch section |
| TX LAMP (white/amber) | Channel actively transmitting — stays lit, brighter than LEDs | TX Lamp (dedicated) |

Key status combinations at a glance:

- BLUE + GREEN = Two-way active — I hear them, they hear me

- BLUE + RED = Monitor mode — I hear them, they cannot hear me

- RED + GREEN = Blind TX — transmitting but deaf to response (avoid)

- RED + RED = Both muted — channel silent in both directions

- YELLOW + BLUE = MON position — speaker on, mic physically locked out

- No lights = Channel OFF

- TX LAMP lit = Someone is actively transmitting on this channel

# 6. Hardware Architecture

## 6.1 Processing Nodes (3x RP2350B)

| Node | Role | Responsibilities |
| --- | --- | --- |
| Master Node (Chip 3) | USB HID + UART Hub | USB connection to PC, UART hub between worker chips, SRS software interface, OBS Websocket integration, system state management, remote firmware flashing controller |
| Worker Node 1 (Chip 1) | OLED Display Manager | All 6 OLED displays, I2C bus management (3 buses), display state tracking |
| Worker Node 2 (Chip 2) | Button & LED Worker | All physical buttons and switches, all LEDs, TX Lamp control, encoder inputs, interrupt-driven instant response |

## 6.2 Architecture Rationale

Dividing responsibilities across three chips follows proven SC-UCC architecture principles:

- Each chip has one job — failure is isolated to one subsystem

- OLEDs glitching does not affect button response

- Button chip locking up does not freeze displays

- Each subsystem can be tested and debugged independently

- Firmware can be updated per-chip without affecting others

- Same toolchain, same UART protocol, same patterns as SC-UCC

## 6.3 Dedicated Power Supply

CHANGE v1→v2: USB power alone is insufficient. A dedicated 5V supply is required.

SC-VOX draws significant current across all active components:

| Component | Count | Approx. Current Draw |
| --- | --- | --- |
| RP2350B @ 5V | 3 | ~150mA each = 450mA |
| SSD1306 OLED @ 3.3V | 6 | ~20mA each = 120mA |
| LEDs (all types) | ~40 | ~10mA each = 400mA |
| TX Lamps (brighter) | 6 | ~30mA each = 180mA |
| Encoders, switches | Various | ~50mA total |
| TOTAL ESTIMATED |  | ~1.2A @ 5V = ~6W minimum |

NOTE: USB 2.0 provides 500mA maximum. USB 3.0 provides 900mA. Neither is sufficient for full SC-VOX load. A dedicated 5V/2A wall adapter or shared PDB with SC-UCC is required.

Recommended power architecture:

- Input: 5V/2A dedicated supply (USB-C PD or barrel jack adapter)

- Rail 1: 5V → all 3x RP2350B logic power

- Rail 2: 3.3V LDO → all 6x OLED displays

- Rail 3: 5V current-limited → all LEDs and TX Lamps (resistor per LED)

- Optional: share PDB with SC-UCC if panels are co-located

## 6.4 Internal Connectivity

- Topology: Star — Master node to worker nodes

- Interconnect: 8-wire shielded backplane harness

- Bus Protocol: UART at 4Mbps with packet framing

- Packet format: [START 0xAA | CMD | PAYLOAD | CRC | END 0x55]

## 6.5 Backplane Harness — 8 Wire

| Wire | Signal | Purpose |
| --- | --- | --- |
| 1 | 5V | Logic power |
| 2 | 3.3V | OLED power |
| 3 | GND | Common ground |
| 4 | UART_TX | Master to workers |
| 5 | UART_RX | Workers to master |
| 6 | BOOT | Remote flash trigger |
| 7 | RESET | Remote reboot control |
| 8 | IRQ | Interrupt / event line |

# 7. GPIO Budget

## 7.1 Per Full Channel GPIO (CH 1-4)

CHANGE v1→v2: Activity flash LED removed. TX Lamp replaces it — same GPIO count (1 GPIO), different component.

| Control | GPIO Count |
| --- | --- |
| 3-position switch | 2 |
| Mic mute button | 1 |
| Speaker mute button | 1 |
| PTT/VOX toggle | 1 |
| Flip-flop button | 1 |
| Volume encoder A/B | 2 |
| Volume encoder push | 1 |
| MON LED (yellow) | 1 |
| ACTIVE LED (green) | 1 |
| Mic GREEN LED | 1 |
| Mic RED LED | 1 |
| Speaker BLUE LED | 1 |
| Speaker RED LED | 1 |
| TX Lamp (replaces activity flash LED) | 1 |
| TOTAL per full channel | 16 |

## 7.2 Total GPIO Summary

| Subsystem | GPIO |
| --- | --- |
| CH 1-4 full channels (16 x 4) | 64 |
| CH 5 Guard (MON LED + TX Lamp) | 2 |
| CH 6 Local (mic/speaker buttons + 5 LEDs + TX Lamp) | 8 |
| I2C buses for 6 OLEDs (3 buses x 2 GPIO) | 6 |
| Master: UART TX/RX + volume encoder + PTT + status LED | 7 |
| TOTAL | 87 |

NOTE: GPIO count unchanged from v1. TX Lamp replaces activity flash LED at the same GPIO cost. Total remains 87 GPIO across 3x RP2350B chips.

# 8. Component Specifications

CHANGE v1→v2: Activity flash LED removed from component list. TX Lamp added as dedicated brighter indicator.

| Component | Spec | Qty | Notes |
| --- | --- | --- | --- |
| Microcontroller | Raspberry Pi RP2350B | 3 | 48 GPIO, dual-core, PIO support |
| OLED Display | 0.91" 128x32 I2C SSD1306 | 6 | 3.3V, I2C address 0x3C/0x3D |
| 3-Position Switch | ON-OFF-ON momentary | 4 | CH 1-4 only |
| PTT/VOX Toggle | Latched toggle switch | 4 | CH 1-4 only |
| Flip-Flop Button | Momentary pushbutton | 4 | CH 1-4 only |
| Mic Mute Button | Momentary pushbutton | 5 | CH 1-4 + CH 6 |
| Speaker Mute Button | Momentary pushbutton | 5 | CH 1-4 + CH 6 |
| Volume Encoder | Rotary encoder + push | 4 | CH 1-4 only |
| Green LED | 3mm or 5mm | 9 | Mic live + Active switch |
| Red LED | 3mm or 5mm | 10 | Mic muted + Speaker muted |
| Blue LED | 3mm or 5mm | 5 | Speaker active |
| Yellow LED | 3mm or 5mm | 6 | MON mode (all channels) |
| TX Lamp | T1-3/4 or panel lamp, white/amber, high brightness | 6 | Sustained TX indicator — one per channel, brighter than standard LEDs |
| QSPI Flash | 16MB per worker node | 2 | Firmware + display assets |
| Buck Converter | Input 5V-15V → 5V output, 2A | 1 | Logic power rail |
| LDO | 5V → 3.3V, high current | 1 | OLED + LED power rail |
| Power Supply | 5V/2A dedicated adapter | 1 | Wall adapter or shared SC-UCC PDB |

# 9. Firmware & Software Architecture

## 9.1 UART Packet Protocol

All inter-chip communication uses framed UART packets at 4Mbps:

[0xAA | CMD | PAYLOAD (1-4 bytes) | CRC | 0x55]

- START byte 0xAA and END byte 0x55 provide resync anchors

- CRC (XOR) catches corruption

- On bad packet: flush buffer, wait for next 0xAA

- Heartbeat from master every 100ms — workers resync on receive

## 9.2 State Machine

| Event | Source | Action |
| --- | --- | --- |
| Switch position changed | Chip 2 | Chip 2 → UART → Master → Chip 1 updates OLED |
| Button pressed | Chip 2 | Chip 2 → UART → Master → sends SRS/VOIP keybind to PC |
| TX active detected | Master (SRS) | Master → UART → Chip 2 lights TX Lamp (sustained) |
| TX ended | Master (SRS) | Master → UART → Chip 2 extinguishes TX Lamp |
| Channel state changed | Master | Master → UART → Chip 1 updates OLED display |
| OBS track state sync | Master | Master → OBS Websocket → mute/unmute audio source |
| Firmware update requested | PC utility | Master pulls BOOT+RESET on target chip → flashes |

## 9.3 Remote Firmware Update

All three chips support remote firmware flashing without opening the enclosure:

- PC utility connects via USB to Master node

- User selects target chip (OLED / Button / Master)

- Master pulls BOOT + RESET pins on target chip via backplane

- Target chip presents as USB mass storage device

- New firmware file is written automatically

- Master releases RESET — chip reboots with new firmware

- No screwdriver, no disassembly required

## 9.4 SRS Integration & Licensing

CHANGE v1→v2: SRS licensing clarified. Fork is permitted under MIT open source license.

Simple Radio Standalone (SRS) is fully open source under MIT license, maintained by ciribob on GitHub. The project has 146+ forks and an active contributor community.

Licensing summary:

- ✅ Fork the source code freely

- ✅ Modify for SC-VOX specific integration

- ✅ Redistribute your fork publicly or keep private

- ✅ Use commercially

- ⚠️ Must retain original copyright notice in derived works

A Star Citizen specific SRS fork already exists (SKWK org). Precedent is established for SC-specific SRS forks.

Recommended approach for SC-VOX:

- Fork the SRS repository

- Strip DCS-specific aircraft radio simulation layer

- Replace with SC channel management (Wing / Squad / Org / Multi-Org / Guard / Local)

- Add SC-VOX hardware API — expose channel state, TX status, frequency to SC-VOX Master node

- Add OBS Websocket integration hooks

- Maintain as SC-VOX-SRS — a purpose-built SC comms client

NOTE: Running a forked SRS client requires running your own SRS server. This is trivial — the SRS server is a lightweight standalone executable that any org member can host.

## 9.5 OBS Websocket Integration

SC-VOX Master node connects to OBS via OBS Websocket plugin:

- Channel mute state changes automatically update corresponding OBS audio source

- Channel active state triggers OBS audio source enable/disable

- Guard channel always enabled in OBS — cannot be muted from hardware

- My Mic and Game Audio tracks managed directly in OBS — not via SC-VOX

# 10. Joystick PTT Integration

All PTT controls live on the Virpil Constellation Alpha Prime HOSAS setup — never requiring the pilot to reach for the SC-VOX panel to transmit:

| Channel | PTT Location | Rationale |
| --- | --- | --- |
| CH 1 Wing | Left D-Pad 2 Up | Left thumb — always accessible |
| CH 2 Squad | Left D-Pad 2 Down | Left thumb — always accessible |
| CH 3 Org | Left D-Pad 2 Left | Left thumb — always accessible |
| CH 4 Multi-Org | Left D-Pad 2 Right | Left thumb — always accessible |
| CH 5 Guard | Left D-Pad 2 Push | Center press — deliberate, harder to accidentally key |
| CH 6 Local | Left Pinky Button | Pinky — natural for proximity/local comms |

NOTE: Left hand manages all comms PTT. Right hand manages flight and combat. Pilots never cross hands to key a radio.

# 11. Physical Design

## 11.1 Panel Layout Concept

Six channel blocks arranged in a vertical stack or horizontal row, each block containing from top to bottom:

- OLED display — channel name, active/standby frequency

- TX Lamp — large, bright, sustained indicator for active transmission

- 3-position switch (OFF / MON / ACTIVE) with MON and ACTIVE LEDs

- Mic section: button + GREEN/RED LEDs

- Speaker section: button + BLUE/RED LEDs

- Volume encoder

- PTT/VOX toggle

- Flip-flop button

Guard channel block (CH 5) — visually distinct, smaller, permanent GUARD label on OLED, yellow MON LED always lit, TX Lamp only.

Local channel block (CH 6) — similar to Guard in size but includes mic and speaker mute buttons and TX Lamp.

## 11.2 Enclosure

- Separate standalone box — not integrated into SC-UCC enclosure

- Same modular subpanel approach as SC-UCC

- Connected to SC-UCC PDB via shared power harness if co-located

- USB connection independent — appears as separate HID device to PC

# 12. Future Considerations

| Feature | Notes |
| --- | --- |
| SC two-way telemetry | If SC adds two-way API, SC-VOX can auto-sync channel state to in-game comms |
| Additional channels | Architecture supports expansion — add worker nodes for more channels |
| Encryption indicators | Military-style SEC/UNSEC LED per channel |
| Dual watch | Monitor two channels simultaneously — software feature, no hardware change |
| Hot mic per channel | Open mic toggle without PTT — crew intercom style |
| SC-ECC integration | Future emote console can share backplane and OBS Websocket connection |
| SC-VOX-SRS fork | Purpose-built SC comms client based on forked SRS codebase |

# 13. Project Status

| Item | Status |
| --- | --- |
| System concept and scope | ✅ Complete |
| Channel definitions | ✅ Complete |
| Feature matrix per channel | ✅ Complete v1.1 |
| LED color scheme | ✅ Complete v1.1 |
| TX Lamp specification | ✅ Complete v1.1 |
| Hardware architecture (3x RP2350B) | ✅ Complete |
| Dedicated power supply spec | ✅ Complete v1.1 |
| GPIO budget | ✅ Complete |
| Component specifications | ✅ Draft v1.1 |
| Joystick PTT mapping | ✅ Complete |
| UART packet protocol | ✅ Defined (shared with SC-UCC) |
| SRS licensing confirmed | ✅ Complete v1.1 — MIT, fork permitted |
| SRS integration spec | ⏳ Pending |
| SC-VOX-SRS fork planning | ⏳ Pending |
| OBS Websocket integration spec | ⏳ Pending |
| PCB / schematic design | ⏳ Pending |
| Enclosure physical design | ⏳ Pending |
| Firmware development | ⏳ Pending |
| Integration with SC-UCC | ⏳ Pending |