SC-VOX

Star Citizen Voice Operations eXchange

Project Specification — Version 1.3

REVISION SUMMARY v1.2 → v1.3

| Change | Detail |
| --- | --- |
| Track mapping revised | CH 1-7 map directly to Ardour Tracks 1-7. My Mic = Track 8. Game Audio = Track 9. |
| OBS tracks defined | 6 OBS tracks specified: My Mic, Game Audio, Wing, Org, Multi-Org, Guard (muted by default) |
| Game Audio as sync channel | Game Audio (WASAPI Loopback) is the primary sync reference between OBS and Ardour |
| Audio routing architecture | WASAPI Shared mode allows OBS and Ardour to simultaneously access same sources — no VAC needed |
| Post-production workflow | Defined: sync via Game Audio reference track, mute OBS audio, use Ardour tracks for final mix |

# 1. System Overview

SC-VOX is a dedicated hardware voice communications panel designed for Star Citizen multi-crew and multi-org operations. It provides physical, tactile control over up to 7 simultaneous communication channels via Simple Radio Standalone (SRS), in-game VOIP, and RSI Spectrum (future), with real-time LED status readout and per-channel audio management.

The panel is designed to complement the SC-UCC (Star Citizen Ultra-Cockpit Controller) button box, sharing the same distributed processing architecture, UART communication protocol, and remote firmware update system.

OPEN SOURCE: SC-VOX targets a 100% open source software stack: OBS Studio (video), Ardour DAW (audio), SRS fork (comms), custom firmware (hardware). No paid software required.

NOTE: SC-VOX is inspired by real aircraft Audio Control Panels (ACP) and military radio stack designs. Guard channel behavior follows real aviation doctrine — fixed frequency, always monitoring, cannot be disabled.

# 2. Design Philosophy

## 2.1 Real Aviation Doctrine

SC-VOX borrows directly from real aircraft comms panel design:

- Guard channel is fixed, always-on, and non-configurable — exactly as in real aviation

- Per-channel OFF / MON / ACTIVE positional switches mirror real ACP design

- Separate mic and speaker mute controls per channel

- Standby frequency pre-tuning with flip-flop swap

- TX Lamp illuminates and stays lit while a channel is actively transmitting

- PTT/VOX toggle per channel for flexibility between formal and casual nets

## 2.2 Immersion First

Every design decision prioritizes immersion and realistic operational feel:

- Guard channel PTT lives on a dedicated joystick button — never on the panel

- Local/proximity channel uses in-game VOIP bindings — not SRS

- No recording controls on hardware — Ardour and OBS handle all recording

- CH 7 Spectrum hardware installed but software disabled — future-proofed for RSI Spectrum voice launch

## 2.3 Open Source Stack

OPEN SOURCE: Every software component in the SC-VOX ecosystem is free and open source.

| Component | Software | License | Purpose |
| --- | --- | --- | --- |
| Video capture / streaming | OBS Studio | GPL v2 | Screen capture, streaming, sync reference audio |
| Audio recording | Ardour DAW | GPL v2 | Unlimited multitrack isolated channel recording |
| Voice comms | SRS (forked) | MIT | Multi-channel radio simulation |
| Hardware firmware | Custom (RP2350B) | MIT (planned) | SC-VOX panel control |
| Hardware interface | Custom PC app | MIT (planned) | Bridge between SRS and SC-VOX hardware |

## 2.4 Audio Routing Architecture

CHANGE v1.2→v1.3: WASAPI Shared mode allows OBS and Ardour to access the same sources simultaneously. No Virtual Audio Cable required.

All audio sources use Windows WASAPI Shared mode — multiple applications can read the same device simultaneously:

| Source | Type | WASAPI Mode | Ardour | OBS |
| --- | --- | --- | --- | --- |
| Microphone | Physical input | Shared | ✅ Track 8 | ✅ Track 1 |
| SRS CH 1-7 | Application audio | Shared | ✅ Tracks 1-7 | ✅ Tracks 3-6 (selected) |
| Game Audio (SC) | WASAPI Loopback | Shared | ✅ Track 9 | ✅ Track 2 |

NOTE: WASAPI Shared mode: In Windows Sound Settings → device Properties → Advanced, uncheck 'Allow applications to take exclusive control'. This forces all apps to share access to that device.

## 2.5 Ardour Track Mapping

CHANGE v1.2→v1.3: CH 1-7 map directly to Ardour Tracks 1-7. My Mic moved to Track 8. Game Audio = Track 9.

| Ardour Track | Source | Input Method | Status |
| --- | --- | --- | --- |
| Track 1 | CH 1 Wing (SRS) | WASAPI Shared — SRS audio output | ✅ Active |
| Track 2 | CH 2 Squad (SRS) | WASAPI Shared — SRS audio output | ✅ Active |
| Track 3 | CH 3 Org (SRS) | WASAPI Shared — SRS audio output | ✅ Active |
| Track 4 | CH 4 Multi-Org (SRS) | WASAPI Shared — SRS audio output | ✅ Active |
| Track 5 | CH 5 Guard (SRS) | WASAPI Shared — SRS audio output | ✅ Active — always recording |
| Track 6 | CH 6 Local (In-game VOIP) | WASAPI Shared — SC game audio | ✅ Active |
| Track 7 | CH 7 Spectrum (RSI) | WASAPI Shared — Spectrum audio | ⏳ Reserved — software disabled |
| Track 8 | My Mic | WASAPI Shared — physical mic input | ✅ Active — always recording |
| Track 9 | Game Audio (SC) | WASAPI Loopback — SC game output | ✅ Active — sync reference |

## 2.6 OBS Track Mapping

CHANGE v1.2→v1.3: OBS tracks defined. Game Audio is primary sync reference. Most comms tracks muted by default for stream.

| OBS Track | Source | Input Method | Default | Purpose |
| --- | --- | --- | --- | --- |
| Track 1 | My Mic | WASAPI Shared — physical mic | Live | Pilot voice — always on stream |
| Track 2 | Game Audio (SC) | WASAPI Loopback | Live | PRIMARY SYNC REFERENCE + game audio for stream |
| Track 3 | CH 1 Wing | WASAPI Shared — SRS output | Muted | Available for stream if desired |
| Track 4 | CH 3 Org | WASAPI Shared — SRS output | Muted | Available for stream if desired |
| Track 5 | CH 4 Multi-Org | WASAPI Shared — SRS output | Muted | Available for stream if desired |
| Track 6 | CH 5 Guard | WASAPI Shared — SRS output | Muted | Available for stream if desired |

SYNC: Game Audio (OBS Track 2 / Ardour Track 9) is the sync reference. Both apps receive the same WASAPI Loopback signal simultaneously. Align these tracks in post to sync all Ardour channels to OBS video.

NOTE: Squad (CH 2) and Local (CH 6) are intentionally excluded from OBS — squad comms may contain sensitive org information, local/proximity is ambient noise not suited for stream.

## 2.7 Post-Production Sync Workflow

Game Audio is the perfect sync reference because it is:

- Continuous — always playing during a session

- Distinctive — SC has unique audio events: explosions, UI sounds, engine hum

- Never silent — unlike mic which can go quiet for long periods

- Frame accurate — same WASAPI Loopback source hitting both apps simultaneously

Step by step post-production workflow:

- 1. Open video editor — import OBS video file

- 2. Import Ardour session — all 9 tracks as separate audio files

- 3. Align Ardour Track 9 (Game Audio) to OBS Track 2 (Game Audio) — they are identical signals

- 4. Lock all Ardour tracks to video — they are now perfectly synced

- 5. Mute OBS audio entirely — Ardour is now the audio source

- 6. Select which comms tracks appear per scene in the final edit

- 7. Mute irrelevant chatter, feature specific channel moments independently

NOTE: This is the same workflow professional filmmakers use — a clapperboard creates a sync point between camera audio and dedicated audio recorder. Game Audio is your clapperboard.

# 3. Channel Definitions

| CH | Net Name | System | Ardour Track | OBS Track | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Wing | SRS | 1 | 3 (muted) | ✅ Active |
| 2 | Squad | SRS | 2 | — (excluded) | ✅ Active |
| 3 | Org | SRS | 3 | 4 (muted) | ✅ Active |
| 4 | Multi-Org | SRS | 4 | 5 (muted) | ✅ Active |
| 5 | GUARD | SRS | 5 | 6 (muted) | ✅ Active — always on |
| 6 | Local | In-game VOIP | 6 | — (excluded) | ✅ Active |
| 7 | Spectrum | RSI Spectrum | 7 (reserved) | — (excluded) | ⏳ Hardware ready — software disabled |
| — | My Mic | Physical input | 8 | 1 (live) | ✅ Active — always recording |
| — | Game Audio | WASAPI Loopback | 9 | 2 (live) | ✅ SYNC REFERENCE |

# 4. Per-Channel Feature Matrix

| Feature | CH 1-4 (Full) | CH 5 Guard | CH 6 Local | CH 7 Spectrum |
| --- | --- | --- | --- | --- |
| OLED Display | ✅ Active/Standby freq | ✅ GUARD (static) | ✅ LOCAL (static) | ✅ SPECTRUM — PENDING |
| 3-Position Switch | ✅ | ❌ | ❌ | ✅ (installed, disabled) |
| PTT / VOX Toggle | ✅ | ❌ PTT only | ❌ PTT only | ✅ (installed, disabled) |
| Flip-Flop Button | ✅ | ❌ | ❌ | ✅ (installed, disabled) |
| Volume Encoder + Push | ✅ | ❌ | ❌ | ✅ (installed, disabled) |
| Mic Mute Button | ✅ | ❌ | ✅ | ✅ (installed, disabled) |
| Speaker Mute Button | ✅ | ❌ | ✅ | ✅ (installed, disabled) |
| MON LED (Yellow) | ✅ | ✅ Always lit | ✅ Always lit | ✅ (installed, off) |
| ACTIVE LED (Green) | ✅ | ❌ | ❌ | ✅ (installed, off) |
| Mic Live LED (Green) | ✅ | ❌ | ✅ | ✅ (installed, off) |
| Mic Muted LED (Red) | ✅ | ❌ | ✅ | ✅ (installed, off) |
| Speaker Active LED (Blue) | ✅ | ❌ | ✅ | ✅ (installed, off) |
| Speaker Muted LED (Red) | ✅ | ❌ | ✅ | ✅ (installed, off) |
| TX Lamp | ✅ | ✅ | ✅ | ✅ (installed, off) |
| Ardour Track | 1, 2, 3, 4 | 5 | 6 | 7 (reserved) |
| OBS Track | 3, —, 4, 5 | 6 | — | — |
| PTT Location | Left D-Pad 2 | Left D-Pad 2 Push | Left Pinky | Reserved (null) |

# 5. LED Color Scheme

| LED Color | Meaning | Location |
| --- | --- | --- |
| GREEN (mic) | Mic live / active — they can hear you | Mic section |
| RED (mic) | Mic muted — cannot transmit | Mic section |
| BLUE | Speaker active — you can hear them | Speaker section |
| RED (speaker) | Speaker muted — deaf to channel | Speaker section |
| YELLOW | MON mode — listening only, TX locked | Switch section |
| GREEN (switch) | ACTIVE mode — full participation | Switch section |
| TX LAMP (white/amber) | Channel actively transmitting — sustained, brighter than LEDs | TX Lamp (dedicated) |

Key status combinations at a glance:

- BLUE + GREEN = Two-way active — I hear them, they hear me

- BLUE + RED = Monitor mode — I hear them, they cannot hear me

- RED + GREEN = Blind TX — transmitting but deaf to response (avoid)

- RED + RED = Both muted — channel silent in both directions

- YELLOW + BLUE = MON position — speaker on, mic physically locked out

- No lights = Channel OFF (or CH 7 software disabled)

- TX LAMP lit = Someone is actively transmitting on this channel

# 6. Hardware Architecture

## 6.1 Processing Nodes (3x RP2350B)

| Node | Role | Responsibilities |
| --- | --- | --- |
| Master Node (Chip 3) | USB HID + UART Hub | USB to PC, UART hub, SRS interface, Ardour OSC integration, system state, remote firmware flashing |
| Worker Node 1 (Chip 1) | OLED Display Manager | All 7 OLED displays, I2C bus management (4 buses), display state, CH 7 PENDING display |
| Worker Node 2 (Chip 2) | Button & LED Worker | All physical buttons including CH 7 (disabled in firmware), all LEDs, TX Lamp control, encoder inputs |

## 6.2 Dedicated Power Supply

| Component | Count | Approx. Current Draw |
| --- | --- | --- |
| RP2350B @ 5V | 3 | ~150mA each = 450mA |
| SSD1306 OLED @ 3.3V | 7 | ~20mA each = 140mA |
| LEDs (all types) | ~46 | ~10mA each = 460mA |
| TX Lamps (brighter) | 7 | ~30mA each = 210mA |
| Encoders, switches | Various | ~50mA total |
| TOTAL ESTIMATED |  | ~1.3A @ 5V = ~7W minimum |

NOTE: USB power alone is insufficient. A dedicated 5V/2A wall adapter or shared PDB with SC-UCC is required.

- Rail 1: 5V → all 3x RP2350B logic power

- Rail 2: 3.3V LDO → all 7x OLED displays

- Rail 3: 5V current-limited → all LEDs and TX Lamps

- Optional: share PDB with SC-UCC if panels are co-located

## 6.3 Internal Connectivity

- Topology: Star — Master node to worker nodes

- Interconnect: 8-wire shielded backplane harness

- Bus Protocol: UART at 4Mbps with packet framing

- Packet format: [START 0xAA | CMD | PAYLOAD | CRC | END 0x55]

## 6.4 Backplane Harness — 8 Wire

| Wire | Signal | Purpose |
| --- | --- | --- |
| 1 | 5V | Logic power |
| 2 | 3.3V | OLED power |
| 3 | GND | Common ground |
| 4 | UART_TX | Master to workers |
| 5 | UART_RX | Workers to master |
| 6 | BOOT | Remote flash trigger |
| 7 | RESET | Remote reboot control |
| 8 | IRQ | Interrupt / event line |

# 7. GPIO Budget

## 7.1 Per Full Channel GPIO (CH 1-4 + CH 7)

| Control | GPIO Count |
| --- | --- |
| 3-position switch | 2 |
| Mic mute button | 1 |
| Speaker mute button | 1 |
| PTT/VOX toggle | 1 |
| Flip-flop button | 1 |
| Volume encoder A/B | 2 |
| Volume encoder push | 1 |
| MON LED (yellow) | 1 |
| ACTIVE LED (green) | 1 |
| Mic GREEN LED | 1 |
| Mic RED LED | 1 |
| Speaker BLUE LED | 1 |
| Speaker RED LED | 1 |
| TX Lamp | 1 |
| TOTAL per full channel | 16 |

## 7.2 Total GPIO Summary

| Subsystem | GPIO |
| --- | --- |
| CH 1-4 full channels (16 x 4) | 64 |
| CH 5 Guard (MON LED + TX Lamp) | 2 |
| CH 6 Local (mic/speaker buttons + 5 LEDs + TX Lamp) | 8 |
| CH 7 Spectrum (16 — same as full channel, software disabled) | 16 |
| I2C buses for 7 OLEDs (4 buses x 2 GPIO) | 8 |
| Master: UART TX/RX + volume encoder + PTT + status LED | 7 |
| TOTAL | 105 |
| AVAILABLE (3x RP2350B @ 48 GPIO) | 144 |
| HEADROOM | 39 |

# 8. Component Specifications

| Component | Spec | Qty | Notes |
| --- | --- | --- | --- |
| Microcontroller | Raspberry Pi RP2350B | 3 | 48 GPIO, dual-core, PIO support |
| OLED Display | 0.91" 128x32 I2C SSD1306 | 7 | 3.3V — CH 1-7 including Spectrum |
| 3-Position Switch | ON-OFF-ON momentary | 5 | CH 1-4 + CH 7 (wired, firmware disabled) |
| PTT/VOX Toggle | Latched toggle switch | 5 | CH 1-4 + CH 7 |
| Flip-Flop Button | Momentary pushbutton | 5 | CH 1-4 + CH 7 |
| Mic Mute Button | Momentary pushbutton | 6 | CH 1-4 + CH 6 + CH 7 |
| Speaker Mute Button | Momentary pushbutton | 6 | CH 1-4 + CH 6 + CH 7 |
| Volume Encoder | Rotary encoder + push | 5 | CH 1-4 + CH 7 |
| Green LED | 3mm or 5mm | 10 | Mic live + Active switch |
| Red LED | 3mm or 5mm | 12 | Mic muted + Speaker muted |
| Blue LED | 3mm or 5mm | 6 | Speaker active |
| Yellow LED | 3mm or 5mm | 7 | MON mode — all 7 channels |
| TX Lamp | T1-3/4 panel lamp, white/amber, high brightness | 7 | One per channel including CH 7 |
| QSPI Flash | 16MB per worker node | 2 | Firmware + display assets |
| Buck Converter | Input 5V-15V → 5V output, 2A | 1 | Logic power rail |
| LDO | 5V → 3.3V, high current | 1 | OLED + LED power rail |
| Power Supply | 5V/2A dedicated adapter | 1 | Wall adapter or shared SC-UCC PDB |

# 9. Firmware & Software Architecture

## 9.1 UART Packet Protocol

[0xAA | CMD | PAYLOAD (1-4 bytes) | CRC | 0x55]

- START byte 0xAA and END byte 0x55 provide resync anchors

- CRC (XOR) catches corruption

- On bad packet: flush buffer, wait for next 0xAA

- Heartbeat from master every 100ms — workers resync on receive

## 9.2 CH 7 Software Disable

bool ch7_spectrum_enabled = false;  // Set true when RSI Spectrum voice launches

- Master ignores all CH 7 button events when false

- CH 7 OLED shows SPECTRUM — PENDING

- All CH 7 LEDs off

- CH 7 PTT joystick button binding returns null

- Enable requires Master node firmware update only — no hardware changes

## 9.3 State Machine

| Event | Source | Action |
| --- | --- | --- |
| Switch position changed | Chip 2 | Chip 2 → UART → Master → Chip 1 updates OLED |
| Button pressed | Chip 2 | Chip 2 → UART → Master → sends SRS/VOIP keybind to PC |
| TX active detected | Master (SRS) | Master → UART → Chip 2 lights TX Lamp (sustained) |
| TX ended | Master (SRS) | Master → UART → Chip 2 extinguishes TX Lamp |
| Ardour track sync | Master | Master → Ardour OSC → mute/unmute audio track |
| CH 7 event received | Chip 2 | Master checks ch7_spectrum_enabled — ignores if false |
| Firmware update | PC utility | Master pulls BOOT+RESET on target chip → flashes |

## 9.4 Remote Firmware Update

- PC utility connects via USB to Master node

- User selects target chip (OLED / Button / Master)

- Master pulls BOOT + RESET pins on target chip via backplane

- Target chip presents as USB mass storage device

- New firmware drops automatically

- Master releases RESET — chip reboots with new firmware

- No screwdriver, no disassembly required

## 9.5 SRS Integration & Licensing

- ✅ MIT open source — fork permitted

- ✅ Strip DCS-specific layer, replace with SC channel management

- ✅ Add SC-VOX hardware API for channel state and TX status

- ✅ Add Ardour OSC integration

- ⚠️  Requires running your own SRS server — lightweight standalone executable

## 9.6 Ardour OSC Integration

Ardour communicates with SC-VOX Master node via OSC (Open Sound Control):

- Channel mute on panel → Ardour track mute via OSC

- Channel unmute on panel → Ardour track unmute via OSC

- Guard channel (Track 5) always unmuted — hardware cannot mute it

- CH 7 Track 7 always empty until Spectrum voice enabled

- Ardour has built-in OSC support — no plugins required

# 10. Joystick PTT Integration

| Channel | PTT Location | Rationale |
| --- | --- | --- |
| CH 1 Wing | Left D-Pad 2 Up | Left thumb — always accessible |
| CH 2 Squad | Left D-Pad 2 Down | Left thumb — always accessible |
| CH 3 Org | Left D-Pad 2 Left | Left thumb — always accessible |
| CH 4 Multi-Org | Left D-Pad 2 Right | Left thumb — always accessible |
| CH 5 Guard | Left D-Pad 2 Push | Center press — deliberate, harder to accidentally key |
| CH 6 Local | Left Pinky Button | Pinky — natural for proximity/local comms |
| CH 7 Spectrum | Reserved stick button (NULL) | Unbound until Spectrum voice launches |

NOTE: Left hand manages all comms PTT. Right hand manages flight and combat. Pilots never cross hands to key a radio.

# 11. Physical Design

## 11.1 Panel Layout

- CH 1-4: Full channel blocks — all controls

- CH 5 Guard: Minimal block — OLED, TX Lamp, MON LED only

- CH 6 Local: Small block — OLED, TX Lamp, MON LED, mic/speaker mute

- CH 7 Spectrum: Full block — identical to CH 1-4, all controls unlit, OLED shows SPECTRUM — PENDING

NOTE: Suggested: place CH 7 at end of row with subtle visual distinction — e.g. a small amber PENDING indicator above the block to distinguish it from active channels at a glance.

## 11.2 Enclosure

- Separate standalone box — not integrated into SC-UCC enclosure

- USB connection independent — separate HID device to PC

- Optional: share PDB with SC-UCC if co-located

# 12. Future Considerations

| Feature | Notes |
| --- | --- |
| CH 7 Spectrum enable | Single firmware flag — no hardware changes required |
| SC two-way telemetry | Auto-sync channel state if CIG adds two-way API |
| Additional channels | Architecture supports expansion via additional worker nodes |
| Dual watch | Software feature only — no hardware change |
| Hot mic per channel | Software feature only |
| SC-ECC integration | Future emote console shares backplane and Ardour session |
| SC-VOX-SRS fork | Purpose-built SC comms client based on SRS |

# 13. Project Status

| Item | Status |
| --- | --- |
| System concept and scope | ✅ Complete |
| Channel definitions (CH 1-7) | ✅ Complete |
| CH 7 Spectrum — hardware spec | ✅ Complete |
| CH 7 Spectrum — software | ⏳ Pending RSI Spectrum voice launch |
| Feature matrix per channel | ✅ Complete |
| LED color scheme | ✅ Complete |
| TX Lamp specification | ✅ Complete |
| Hardware architecture (3x RP2350B) | ✅ Complete |
| Dedicated power supply spec | ✅ Complete |
| GPIO budget (105 GPIO, 39 headroom) | ✅ Complete |
| Component specifications | ✅ Draft |
| Ardour track mapping (Tracks 1-9) | ✅ Complete v1.3 |
| OBS track mapping (Tracks 1-6) | ✅ Complete v1.3 |
| Game Audio sync workflow | ✅ Complete v1.3 |
| WASAPI Shared audio routing | ✅ Complete v1.3 |
| Post-production sync workflow | ✅ Complete v1.3 |
| Joystick PTT mapping | ✅ Complete |
| Open source stack defined | ✅ Complete — OBS + Ardour + SRS fork |
| Ardour OSC integration spec | ⏳ Pending detail |
| SRS fork planning | ⏳ Pending |
| PCB / schematic design | ⏳ Pending |
| Enclosure physical design | ⏳ Pending |
| Firmware development | ⏳ Pending |
| Integration with SC-UCC | ⏳ Pending |