SC-VOX

Star Citizen Voice Operations eXchange

Project Specification — Version 1.0

A dedicated hardware radio comms panel for Star Citizen multi-org operations.

# 1. System Overview

SC-VOX is a dedicated hardware voice communications panel designed for Star Citizen multi-crew and multi-org operations. It provides physical, tactile control over up to 6 simultaneous communication channels via Simple Radio Standalone (SRS), in-game VOIP, and Discord, with real-time LED status readout and per-channel audio management.

The panel is designed to complement the SC-UCC (Star Citizen Ultra-Cockpit Controller) button box, sharing the same distributed processing architecture, UART communication protocol, and remote firmware update system.

NOTE: SC-VOX is inspired by real aircraft Audio Control Panels (ACP) and military radio stack designs. Guard channel behavior follows real aviation doctrine — fixed frequency, always monitoring, cannot be disabled.

# 2. Design Philosophy

## 2.1 Real Aviation Doctrine

SC-VOX borrows directly from real aircraft comms panel design:

- Guard channel is fixed, always-on, and non-configurable — exactly as in real aviation

- Per-channel OFF / MON / ACTIVE positional switches mirror real ACP design

- Separate mic and speaker mute controls per channel

- Standby frequency pre-tuning with flip-flop swap

- Activity flash LED indicates incoming transmission even when speaker is muted

- PTT/VOX toggle per channel for flexibility between formal and casual nets

## 2.2 Immersion First

Every design decision prioritizes immersion and realistic operational feel:

- Guard channel PTT lives on a dedicated joystick button — never on the panel

- Local/proximity channel uses in-game VOIP bindings — not SRS

- No recording controls on hardware — OBS handles all recording and mixing

- Per-channel OBS audio track integration via OBS Websocket

## 2.3 OBS Integration

SC-VOX integrates with OBS Studio for professional multi-track recording:

| OBS Track | Source | Purpose |
| --- | --- | --- |
| Track 1 | My Mic | Always recorded — pilot voice |
| Track 2 | CH 1 Wing | Wing / flight comms |
| Track 3 | CH 2 Squad | Squad comms |
| Track 4 | CH 3 Org | Org command net |
| Track 5 | CH 4 Multi-Org | Cross-org command |
| Track 6 | CH 5 Guard | Guard channel (always on) |
| Track 7 | CH 6 Local | In-game proximity VOIP |
| Track 8 | Game Audio | SC game audio only |

Post-production editors can select which comms tracks appear in the final video, mute irrelevant chatter, and feature specific channel moments independently.

# 3. Channel Definitions

| CH | Net Name | System | Type | Notes |
| --- | --- | --- | --- | --- |
| 1 | Wing | SRS | Full controls | Immediate crew — 2-4 pilots |
| 2 | Squad | SRS | Full controls | Squad tactical net |
| 3 | Org | SRS | Full controls | Organization command net |
| 4 | Multi-Org | SRS | Full controls | Cross-org coordination |
| 5 | GUARD | SRS | Fixed / passive | Always on — cannot be disabled or tuned |
| 6 | Local | In-game VOIP | Mute controls only | Proximity based — no tuning |

# 4. Per-Channel Feature Matrix

| Feature | CH 1-4 (Full) | CH 5 Guard | CH 6 Local |
| --- | --- | --- | --- |
| OLED Display | ✅ Active/Standby freq | ✅ GUARD (static) | ✅ LOCAL (static) |
| 3-Position Switch (OFF/MON/TX) | ✅ | ❌ | ❌ |
| PTT / VOX Toggle | ✅ | ❌ PTT only | ❌ PTT only |
| Flip-Flop Button (Active ↔ Standby) | ✅ | ❌ | ❌ |
| Volume Encoder + Push | ✅ | ❌ | ❌ |
| Mic Mute Button | ✅ | ❌ | ✅ |
| Speaker Mute Button | ✅ | ❌ | ✅ |
| MON LED (Yellow) | ✅ | ✅ Always lit | ✅ Always lit |
| ACTIVE LED (Green) | ✅ | ❌ | ❌ |
| Mic Live LED (Green) | ✅ | ❌ | ✅ |
| Mic Muted LED (Red) | ✅ | ❌ | ✅ |
| Speaker Active LED (Blue) | ✅ | ❌ | ✅ |
| Speaker Muted LED (Red) | ✅ | ❌ | ✅ |
| Activity Flash LED | ✅ | ✅ | ✅ |
| PTT Location | Left D-Pad 2 | Dedicated stick button | Left Pinky button |

# 5. LED Color Scheme

All LEDs follow a consistent color language for instant status readout:

| LED Color | Meaning | Location |
| --- | --- | --- |
| GREEN | Mic live / active — they can hear you | Mic section |
| RED (mic) | Mic muted — cannot transmit | Mic section |
| BLUE | Speaker active — you can hear them | Speaker section |
| RED (speaker) | Speaker muted — deaf to channel | Speaker section |
| YELLOW | MON mode — listening only, TX locked | Switch section |
| GREEN (switch) | ACTIVE mode — full participation | Switch section |
| FLASH | Incoming transmission detected | Activity LED |

Key status combinations at a glance:

- BLUE + GREEN = Two-way active — I hear them, they hear me

- BLUE + RED = Monitor mode — I hear them, they cannot hear me

- RED + GREEN = Blind TX — transmitting but deaf to response (avoid)

- RED + RED = Both muted — channel silent in both directions

- YELLOW + BLUE = MON position — speaker on, mic physically locked out

- No lights = Channel OFF

# 6. Hardware Architecture

## 6.1 Processing Nodes (3x RP2350B)

| Node | Role | Responsibilities |
| --- | --- | --- |
| Master Node (Chip 3) | USB HID + UART Hub | USB connection to PC, UART hub between worker chips, SRS software interface, OBS Websocket integration, system state management, remote firmware flashing controller |
| Worker Node 1 (Chip 1) | OLED Display Manager | All 6 OLED displays, I2C bus management (3 buses), display state tracking, activity flash animation logic |
| Worker Node 2 (Chip 2) | Button & LED Worker | All physical buttons and switches, all LEDs, encoder inputs, interrupt-driven instant response |

## 6.2 Architecture Rationale

Dividing responsibilities across three chips follows proven SC-UCC architecture principles:

- Each chip has one job — failure is isolated to one subsystem

- OLEDs glitching does not affect button response

- Button chip locking up does not freeze displays

- Each subsystem can be tested and debugged independently

- Firmware can be updated per-chip without affecting others

- Same toolchain, same UART protocol, same patterns as SC-UCC

## 6.3 Power Distribution

| Rail | Voltage | Powers |
| --- | --- | --- |
| Rail 1 | 5V (Buck regulated) | All 3 RP2350B logic |
| Rail 2 | 3.3V (LDO) | All 6 OLED displays |
| Rail 3 | 3.3V / 5V | All LEDs (current limited) |

## 6.4 Internal Connectivity

- Topology: Star — Master node to worker nodes

- Interconnect: 8-wire shielded backplane harness

- Bus Protocol: UART at 4Mbps with packet framing

- Packet format: [START 0xAA | CMD | PAYLOAD | CRC | END 0x55]

## 6.5 Backplane Harness — 8 Wire

| Wire | Signal | Purpose |
| --- | --- | --- |
| 1 | 5V | Logic power |
| 2 | 3.3V | OLED power |
| 3 | GND | Common ground |
| 4 | UART_TX | Master to workers |
| 5 | UART_RX | Workers to master |
| 6 | BOOT | Remote flash trigger |
| 7 | RESET | Remote reboot control |
| 8 | IRQ | Interrupt / event line |

# 7. GPIO Budget

## 7.1 Per Full Channel GPIO (CH 1-4)

| Control | GPIO Count |
| --- | --- |
| 3-position switch | 2 |
| Mic mute button | 1 |
| Speaker mute button | 1 |
| PTT/VOX toggle | 1 |
| Flip-flop button | 1 |
| Volume encoder A/B | 2 |
| Volume encoder push | 1 |
| MON LED (yellow) | 1 |
| ACTIVE LED (green) | 1 |
| Mic GREEN LED | 1 |
| Mic RED LED | 1 |
| Speaker BLUE LED | 1 |
| Speaker RED LED | 1 |
| Activity flash LED | 1 |
| TOTAL per full channel | 16 |

## 7.2 Total GPIO Summary

| Subsystem | GPIO |
| --- | --- |
| CH 1-4 full channels (16 x 4) | 64 |
| CH 5 Guard (MON LED + Activity LED) | 2 |
| CH 6 Local (mic/speaker buttons + 5 LEDs + activity) | 8 |
| I2C buses for 6 OLEDs (3 buses x 2 GPIO) | 6 |
| Master: UART TX/RX + volume encoder + PTT + status LED | 7 |
| TOTAL | 87 |

NOTE: RP2350B provides 48 GPIO per chip. 87 total GPIO requires minimum 2 chips. 3 chips are used for architectural cleanliness, reliability, and headroom — not raw GPIO count.

# 8. Component Specifications

| Component | Spec | Qty | Notes |
| --- | --- | --- | --- |
| Microcontroller | Raspberry Pi RP2350B | 3 | 48 GPIO, dual-core, PIO support |
| OLED Display | 0.91" 128x32 I2C SSD1306 | 6 | 3.3V, I2C address 0x3C/0x3D |
| 3-Position Switch | ON-OFF-ON momentary | 4 | CH 1-4 only |
| PTT/VOX Toggle | Latched toggle switch | 4 | CH 1-4 only |
| Flip-Flop Button | Momentary pushbutton | 4 | CH 1-4 only |
| Mic Mute Button | Momentary pushbutton | 5 | CH 1-4 + CH 6 |
| Speaker Mute Button | Momentary pushbutton | 5 | CH 1-4 + CH 6 |
| Volume Encoder | Rotary encoder + push | 4 | CH 1-4 only |
| Green LED | 3mm or 5mm | 9 | Mic live + Active switch |
| Red LED | 3mm or 5mm | 10 | Mic muted + Speaker muted |
| Blue LED | 3mm or 5mm | 5 | Speaker active |
| Yellow LED | 3mm or 5mm | 6 | MON mode (all channels) |
| White/Amber LED | 3mm or 5mm | 6 | Activity flash (all channels) |
| QSPI Flash | 16MB per worker node | 2 | Firmware + display assets |
| Buck Converter | Input 15V → 5V output | 1 | Logic power rail |
| LDO / Buck | 5V → 3.3V high current | 1 | OLED power rail |

# 9. Firmware & Software Architecture

## 9.1 UART Packet Protocol

All inter-chip communication uses framed UART packets at 4Mbps:

[0xAA | CMD | PAYLOAD (1-4 bytes) | CRC | 0x55]

- START byte 0xAA and END byte 0x55 provide resync anchors

- CRC (XOR) catches corruption

- On bad packet: flush buffer, wait for next 0xAA

- Heartbeat from master every 100ms — workers resync on receive

## 9.2 State Machine

| Event | Source | Action |
| --- | --- | --- |
| Switch position changed | Chip 2 | Chip 2 → UART → Master → Chip 1 updates OLED |
| Button pressed | Chip 2 | Chip 2 → UART → Master → sends SRS/VOIP keybind to PC |
| Channel activity detected | Master (SRS) | Master → UART → Chip 2 flashes activity LED |
| Channel state changed | Master | Master → UART → Chip 1 updates OLED display |
| OBS track state sync | Master | Master → OBS Websocket → mute/unmute audio source |
| Firmware update requested | PC utility | Master pulls BOOT+RESET on target chip → flashes |

## 9.3 Remote Firmware Update

All three chips support remote firmware flashing without opening the enclosure:

- PC utility connects via USB to Master node

- User selects target chip (OLED / Button / Master)

- Master pulls BOOT + RESET pins on target chip via backplane

- Target chip presents as USB mass storage device

- New firmware file is written automatically

- Master releases RESET — chip reboots with new firmware

- No screwdriver, no disassembly required

## 9.4 SRS Integration

SC-VOX communicates with Simple Radio Standalone via its built-in external control API:

- Channel frequency tuning via SRS API commands

- PTT activation via SRS API or bound keyboard inputs

- Channel state reading for activity LED updates

- VOX sensitivity per channel via SRS settings

## 9.5 OBS Websocket Integration

SC-VOX Master node connects to OBS via OBS Websocket plugin:

- Channel mute state changes automatically update corresponding OBS audio source

- Channel active state triggers OBS audio source enable/disable

- Guard channel always enabled in OBS — cannot be muted from hardware

- My Mic and Game Audio tracks managed directly in OBS — not via SC-VOX

# 10. Joystick PTT Integration

All PTT controls live on the Virpil Constellation Alpha Prime HOSAS setup — never requiring the pilot to reach for the SC-VOX panel to transmit:

| Channel | PTT Location | Rationale |
| --- | --- | --- |
| CH 1 Wing | Left D-Pad 2 Up | Left thumb — always accessible |
| CH 2 Squad | Left D-Pad 2 Down | Left thumb — always accessible |
| CH 3 Org | Left D-Pad 2 Left | Left thumb — always accessible |
| CH 4 Multi-Org | Left D-Pad 2 Right | Left thumb — always accessible |
| CH 5 Guard | Left D-Pad 2 Push | Center press — deliberate, harder to accidentally key |
| CH 6 Local | Left Pinky Button | Pinky — natural for proximity/local comms |

NOTE: Left hand manages all comms PTT. Right hand manages flight and combat. Pilots never cross hands to key a radio.

# 11. Physical Design

## 11.1 Panel Layout Concept

Six channel blocks arranged in a vertical stack or horizontal row, each block containing from top to bottom:

- OLED display — channel name, active/standby frequency

- 3-position switch (OFF / MON / ACTIVE) with MON and ACTIVE LEDs

- Mic section: button + GREEN/RED LEDs

- Speaker section: button + BLUE/RED LEDs

- Activity flash LED

- Volume encoder

- PTT/VOX toggle

- Flip-flop button

Guard channel block (CH 5) is visually distinct — smaller, no switches, permanent GUARD label on OLED, yellow MON LED always lit.

Local channel block (CH 6) is similar to Guard in size but includes mic and speaker mute buttons.

## 11.2 Enclosure

- Separate standalone box — not integrated into SC-UCC enclosure

- Same modular subpanel approach as SC-UCC

- Connected to SC-UCC PDB via shared power harness if co-located

- USB connection independent — appears as separate HID device to PC

# 12. Future Considerations

| Feature | Notes |
| --- | --- |
| SC two-way telemetry | If Star Citizen adds two-way comms API, SC-VOX can auto-sync channel state to in-game comms |
| Additional channels | Architecture supports expansion — add worker nodes for more channels |
| Encryption indicators | Military-style SEC/UNSEC LED per channel for RP security awareness |
| Dual watch | Monitor two channels simultaneously — software feature, no hardware change required |
| Hot mic per channel | Open mic toggle without PTT — useful for intercom-style crew comms |
| SC-ECC integration | Future emote console can share backplane and OBS Websocket connection |

# 13. Project Status

| Item | Status |
| --- | --- |
| System concept and scope | ✅ Complete |
| Channel definitions | ✅ Complete |
| Feature matrix per channel | ✅ Complete |
| LED color scheme | ✅ Complete |
| Hardware architecture (3x RP2350B) | ✅ Complete |
| GPIO budget | ✅ Complete |
| Component specifications | ✅ Draft |
| Joystick PTT mapping | ✅ Complete |
| UART packet protocol | ✅ Defined (shared with SC-UCC) |
| SRS integration spec | ⏳ Pending |
| OBS Websocket integration spec | ⏳ Pending |
| PCB / schematic design | ⏳ Pending |
| Enclosure physical design | ⏳ Pending |
| Firmware development | ⏳ Pending |
| Integration with SC-UCC | ⏳ Pending |